|  | monnier Posted: 03/31/04 03:32 PM | First cool down of MIRC dewar. After maintaining guard vacuum of 3e-4 mbar overnight, we filled dewar (both inner and outer vessels) with LN2. We will carry out "hold time" test. |
| --- | --- | --- |

|  | monnier Posted: 05/24/04 04:25 PM | Detector Plug-in!. We plugged in Detector 252 on 2004May22. Unfortunately, the readout is funny -- still debugging.... |
| --- | --- | --- |

|  | monnier Posted: 08/13/04 04:49 PM | Re-mounted PICNIC detector with indium foil. We used 0.005" indium foil (99.99%) to re-mount the PICNIC chip in the dewar card. A 19mm square with a few dabs of apiezon N grease was used between "anvil" and back of FPA chip. We used a "picture frame" of indium (1.6mm wide, 20.7mm across end-to-end). |
| --- | --- | --- |

|  | monnier Posted: 08/13/04 04:51 PM | Populated Filter Wheels. Wheel One: OPEN, 0.5mm pinhole, 1.0mm pinhole, GRISM1 filter, GRISM2 filter, CaF2 200mm lens (not quite in installed) Wheel Two (near detector): OPEN, J, H, K, Kshort, PLUG (77K) |
| --- | --- | --- |

|  | monnier Posted: 05/11/05 05:59 PM | Fiber cut to equal lengths. John and Ajay finished cleaving all the fibers to (hopefullY) equal lengths. Both for the H-band telecom v-groove array (all 32 fibers) and also the Highwave custom fiber in the oz-optics v-groove (10 fibers). |
| --- | --- | --- |

|  | monnier Posted: 05/19/05 09:00 AM | First Lab Fringes!!. We obtained first lab fringes at 5pm on May 18, 2005. The data looks beautiful! |
| --- | --- | --- |

|  | monnier Posted: 05/21/05 04:51 PM | First White Light Fringes!. 2005 May 20. We also successfully tests the cylindrical lens and the prisms. They look quite workable. |
| --- | --- | --- |