Request for Quotation:

2007Oct17

This is a simple request for a quote for 16 small spherical mirrors with silver coating +overcoat.  See below for specifications.  Please let me know if you have any problems with specifications since we have some flexibility.

Looking forward to hearing from you,

-John Monnier	 (monnier@umich.edu)

Univ. Michigan

734-763-5822

Number of Pieces: 16

Spherical Concave Mirror (focusing)

Radius of Curvature 900mm  (+/- 1 %)

Diameter of individual pieces: 25.4mm

Clear Aperture 90%

Wavefront Quality: P-V lambda/10 (632mu)

Scratch-dig: 20-10

Coating: Silver + overcoat

Interferograms required for each piece.

If  I’ve not included an important tolerance, then please use standard values and report them in your response.  Thank you.